Specifications for NBS Chorus

VitraDual & VitraFix (VF1/VF2) Cassette Fix System

SUMMARY

System name: VitraDual Cassette Fix Rainscreen System

System basis of design: VitraDual A1 aluminium cassette panels installed on the VitraFix carrier system.

Manufacturer: Valcan with approved partner system components

System type: A1 aluminium cassette fix rainscreen façade

Fixing method: Secret / cassette fix

Fire rating: A1 throughout (panels, carrier, insulation, membrane)

Warranty: Up to 30 years

Typical applications: Residential, commercial, high-rise

Standards: BS 8414 tested, CWCT, BBA certified*

* Vitradual 3mm is manufactured by Valcan and is available with BBA certification when produced at BBA-certified production facilities. Certification requirements must be specified at enquiry and order stage. Contact us for more details

CONTENTS

| VITRADUAL A1 ALUMINIUM CASSETTE RAINSCREEN SYSTEM | P 4-5 |
| --- | --- |
| System description |  |
| Panel description |  |
| Performance |  |
| Finish and colour |  |
| Application |  |
| Specifiers notes – VitraDual | P 6 |
| VITRAFIX ALUMINIUM CARRIER SYSTEM OVERVIEW | P 7-8 |
| System description |  |
| Performance and compliance |  |
| System options |  |
| Application |  |
| TYPICAL DRAWINGS | P 9-11 |
| Cassette fix typical joint |  |
| Cassette fix horizontal joint |  |
| Cassette fix external corner |  |
| Cassette fix internal corner |  |
| Specifiers notes - VitraFix | P 10 |
| TECHNICAL SCHEDULE | P 11 – 15 |
| System summary |  |
| Primary cladding details |  |
| Panel Fixings and stiffening |  |
| Carrier System – Primary Support |  |
| Horizontal rails (if required) |  |
| Joints and tolerances |  |
| Cavity and ventilation |  |
| Ancillary components |  |
| Masonry fixings (if required) |  |
| TECHNICAL DOWNLOADS | P 16 |
| ADDITIONAL RESOURCES | P 17 |
| Contact details | P 18 |

VITRADUAL A1 ALUMINIUM CASSETTE RAINSCREEN SYSTEM

SYSTEM DESCRIPTON

VitraDual is a non-combustible aluminium cassette fix rainscreen façade system, based on folded aluminium cassette panels with concealed fixings, installed onto an aluminium carrier system. The system is designed to provide a flat, durable and architecturally refined façade solution suitable for use on high-rise and low-rise buildings.

The system is eligible for a single-point, system-based warranty when supplied and installed in accordance with manufacturer’s recommendations.

PANEL DESCRIPTION

VitraDual panels are manufactured from solid aluminium and formed as folded cassette units with concealed fixings located within panel reveals or joints.

- Material: Aluminium alloy (5754 marine-grade)

- Thickness:

  - 2.0 mm (cassette fixing only)

  - 3.0 mm

- Fire classification: Classified A1 in accordance with BS EN 13501-1

- Fixing method: Secret-fixed cassette system

- Surface flatness: Extremely flat finish with low surface stress and no oil-canning effect when designed and installed in accordance with manufacturer guidance.

Panels may be manufactured to bespoke sizes and formats, including curved or perforated panels, subject to project requirements and structural assessment.

PERFORMANCE

Fire performance: Non-combustible system suitable for use in buildings subject to ADB v2 requirements.

Wind loading: CWCT tested and accredited to a safety loading of 3600 Pa (subject to project-specific calculations).

Thermal Conductivity: 138 W/mK

Durability: High impact resistance and suitable for use in high-traffic locations.

Certification: 3mm BBA certified* (Certificate No. 20/5824).

Final system performance is subject to project-specific structural calculations and façade design.

* Vitradual 3mm is manufactured by Valcan and is available with BBA certification when produced at BBA-certified production facilities. Certification requirements must be specified at enquiry and order stage. Contact us for more details

FINISH AND COLOUR

Panels are factory finished with high-performance PVDF Kynar 500 or FEVE paint systems, providing excellent resistance to weathering, UV exposure and atmospheric pollutants.

Colour range: Standard and bespoke colours available.

Maintenance: Low maintenance compared to polyester-coated alternatives.

APPLICATION

The VitraDual system is suitable for use on both internal and external façades of commercial, residential and industrial buildings where a clean, modern architectural appearance is required.

Typical applications include:

- High-rise residential buildings

- Commercial offices

- Mixed-use developments

- Areas requiring robust, non-combustible façade materials

Specifiers notes:

General: Specifier to confirm system configuration, panel dimensions and fixing strategy in conjunction with the manufacturer to suit project-specific structural, fire and environmental requirements.

Fire Performance: The VitraDual system is non-combustible and achieves a fire classification of A1 to BS EN 13501‑1. System build-up, including insulation, cavity barriers and breather membrane, shall be coordinated to ensure compliance with Approved Document B (ADB) and project fire strategy.

Panel Size and Layout: Maximum panel dimensions, joint spacing and cassette profiles are subject to wind loading, substrate condition and fixing centres. Final panel sizes shall be confirmed through project-specific calculations and manufacturer approval.

Wind Loading and Structural Design: Bracket type, spacing and panel support shall be determined based on site wind load calculations in accordance with CWCT guidance. Reference wind load testing is to 3600 Pa safety loading; higher or project-specific loadings may require design adjustment.

Finish and Colour Selection: Final colour, gloss level and coating system (PVDF Kynar 500 or FEVE) to be selected from the manufacturer’s standard or bespoke ranges. Colour selection may influence lead times and warranty duration.

Cavity Depth and Fire Barriers: Minimum cavity depths to be maintained in accordance with CWCT guidance, fire barrier requirements and airflow performance. Cavity depth reductions may be required at fire barrier locations and other pinch points and shall be coordinated at detailed design stage.

Thermal Bridging: Thermal isolators or pads may be required at bracket locations to limit cold bridging in accordance with project thermal performance requirements.

Warranty: Manufacturer warranty of up to 30 years is available, subject to system configuration, coating specification and compliance with Valcan installation guidance.

Coordination: Window interfaces, movement joints, flashings and terminations shall be detailed in coordination with adjacent façade elements to ensure continuity of fire performance, weather resistance and visual alignment.

VITRAFIX ALUMINIUM CARRIER SYSTEM OVERVIEW

The following carrier system details describe a typical aluminium support system suitable for cassette-fixed A1 rainscreen façades, illustrated using the VitraFix carrier system.

SYSTEM DESCRIPTION

VitraFix is an aluminium carrier system comprising brackets, rails and fixings designed to support non-combustible rainscreen cladding systems. The system is tested to BS 8414-1 and BS 8414-2 and assessed by the Centre for Window and Cladding Technology (CWCT).

The system is designed to provide a cost-effective, durable and adjustable support solution suitable for use across a wide range of building types and substrates.

PERFORMANCE AND COMPLIANCE

Fire performance: Tested to BS 8414-1 and BS 8414-2 as part of an external wall system.

Structural assessment: CWCT tested (subject to project-specific calculations).

Durability: Aluminium components designed for long-term external use.

Warranty: A system warranty is available when specified and installed in accordance with manufacturer’s requirements.

SYSTEM OPTIONS

VitraFix VF1 – Primary Bracket System

VitraFix VF1 is an adjustable aluminium bracket system designed to accommodate variations in substrate tolerance and cavity depth.

- Adjustability: 102–235 mm

- Application: Direct bracket fixing to structural substrate

- Extensions: Additional depth may be achieved by incorporating the VitraFix VF2 system components.

- Options: Custom bracket depths are available subject to project requirements.

VF1 is typically used as the primary carrier system for cassette-fixed rainscreen façades.

VitraFix VF2 – Rail and Secondary Support System

VitraFix VF2 comprises aluminium or steel top-hats and C-channel rails designed to provide additional system depth or allow brackets to be installed independently of stud locations.

- Enables alignment of the carrier system where stud positions are restrictive

- Can be used to increase overall system build-up depth

- Top-hats may be used independently as an alternative to timber batten sub-grids

VF2 components may be used in conjunction with VF1 brackets or as part of a secondary support arrangement, subject to design requirements.

APPLICATION

The VitraFix carrier system is suitable for use with non-combustible rainscreen façade systems on residential, commercial and mixed-use buildings. Final system selection, bracket spacing and fixing strategy shall be determined by project-specific structural and wind load calculations.

Specifier’s Notes:

Bracket type, spacing and material selection to be confirmed based on substrate, cavity depth and wind load calculations.

Movement joints and tolerance allowances to be incorporated in façade design to accommodate thermal expansion, substrate movement and building deflection.

TYPICAL DETAILS

Details shown are indicative only and shall be developed to suit project‑specific geometry, substrate and fire strategy.

CASSETTE FIX DETAILS SHOWING TYPICAL JOINT

CASSETTE FIX DETAILS SHOWING HORIZONTAL JOINT

CASSETTE FIX DETAILS SHOWING EXTERNAL CORNER

CASSETTE FIX DETAILS SHOWING INTERNAL CORNER

All Technical Details can be found at https://valcan.co.uk/wp-content/uploads/2026/02/VitraDual-Typical-Details-2026.zip or on request.

TECHNICAL SCHEDULE

Purpose of this schedule

This Technical Schedule provides detailed system components and reference information to support detailed design, coordination and installation.

Final configuration, sizing and fixing arrangements shall be confirmed by project-specific calculations and manufacturer approval.

1. SYSTEM SUMMARY

- Façade system name: VitraDual Cassette System

- System type: Aluminium cassette rainscreen façade system

- Primary carrier system: VitraFix VF1 / VF2

- Fire classification (overall system intent): A1

- Applicable standards: CWCT, BBA

(Informational summary only – see sections below for component detail)

2. PRIMARY CLADDING DETAILS

Product reference: VitraDual Cassette Fix

Manufacturer: Fairview Europe t/a Valcan

Product name: VitraDual

Material: 5754 grade aluminium

Thickness and Weight:

- Thickness: 3mm

- Weight (kg/m²): 8.13 kg/m².

Panel Dimensions:

- Indicative maximum panel sizes up to 5000 mm x 1500 mm, subject to wind loading, handling and cassette geometry.

- Depth – 20-40mm deep according to design

Fire Performance:

- Classification to BS EN 13501-1: Classified A1

3. PANEL FIXING AND STIFFENING

3.1 PRIMARY PANEL FIXINGS

|  | Rail/fixing type | Reference | Sizes |
| --- | --- | --- | --- |
|  | VitraFix Self Drill Panhead A4, colour matched to panel | VFSDP5.535C | 5.5x35mm (200 box) |

Panel Stiffening:

- Stiffener type: 2mm aluminium top-hat attached to reverse of panel using stud weld fixings

4. CARRIER SYSTEM

Carrier system reference: VitraFix VF1

- Manufacturer: Fairview Europe t/a Valcan

Material: Aluminium

System Description: Brief description of role within façade build-up

|  | Bracket Reference | Size | Adjustment Range |
| --- | --- | --- | --- |
|  | VF140DB | 65 mm-260 mm double bracket | 142–172 mm |
|  | VF140SB | 65mm-260 mm single bracket | 142–172mm |

4.1 BRACKETS

Bracket selection and spacing subject to structural and wind load calculations.

4.2 THERMAL/ISOLATION PADS

|  | Bracket Reference | Size | Adjustment Range |
| --- | --- | --- | --- |
|  | VFIPDB | Double bracket thermal pad | 3 mm |
|  | VFIPSB | Single bracket thermal pad | 3mm |

4.3 RAILS

|  | Rail type | Reference | Sizes |
| --- | --- | --- | --- |
|  | T-Rail | VFTR503 / VFTR506 | 50 x 100 x 3000 mm / 50 x 100 x 6000 mm |
|  | L-Rail | VFLR503 / VFLR506 | 50 x 50 x 3000 mm / 50 x 50 x 6000 mm |

5. HORIZONTAL RAILS (IF APPLICABLE)

System reference: VitraFix VF2

Purpose: VF2/U attached to SFS locations allowing brackets to be installed in isolation to stud location

Typical Applications: Not required for masonry

5.1 RAILS AND FIXINGS

|  | Rail/fixing type | Reference | Sizes |
| --- | --- | --- | --- |
|  | U-Rail | VF2/U | 90x15x2mm / 174x15x2mm |
|  | VitraFix Self Drill Hex head with washer | VFSD.538/A4 | 5.5x38mm |
|  | VitraFix Self Drill Hex head | VFSD4.825 (See manufacturers details for fixing centres) | 4.8x25mm, 8mm |
|  | Masonry anchors - ETA approved fire-rated fixing | Valcan partner approved fixings |  |

6. JOINTS AND TOLERANCES

Panel Joint Type:

- Typically 20mm Shadow Gap

- Closed joints

7. CAVITY AND VENTILATION

Cavity Depth:

- Minimum of 25 mm closed joint installation - measured from face of insulation to rear of panel, can be reduced by 50% at fire barrier locations or other “pinch” points. (Check with insulation supplier on suitability of cavity depth for their product)

- [other] - consult CWCT section 3.4.4.4

Ventilation Strategy:

- Closed state

- Refer to CWCT guidance where applicable

8. ANCILLARY COMPONENTS

8.1 SHEATHING BOARDS

- Product reference: ProcellaPro

- Panel size: 2500 x 1250mm

- Thickness/weight: 12 mm / 21.6 kg/m²

- Fire Rating to BS EN  13501-1:	A1

- Manufacturer: Fairview Europe t/a Valcan

8.2 FIXINGS

|  | Rail/fixing type | Reference | Sizes |
| --- | --- | --- | --- |
|  | VitraFix Self Drill ProcellaPro WT fixing | VFSDPP4.838/WT | 4.8x38mm |

8.3 FIRE BARRIERS

Product reference:

- Open state horizontal barrier

- Non-ventilated vertical barrier

Fire performance:

- Insulation:

- Integrity:

Manufacturer: Valcan partner manufacturer of fire barriers

Fixings: Fire tested fixings to be advised by Valcan to equivalent performance of fire barrier, fixing reference dictated by substrate.

8.4 BREATHER MEMBRANE

- Product reference: Valcan partner manufacturer Breather Membrane

- Fire performance (BS EN 13501-1): A1 or A2

8.5 INSULATION

- Product reference: Valcan partner manufacturer of insulation

- Thickness: TBC

- Thermal Performance: 0.034 w/mK

- Fire performance (BS EN 13501-1): A1

9. MASONRY FIXINGS (IF REQUIRED)

Masonry / Concrete Fixings:

- ETA-approved, fire-rated fixings

- Pull-out testing to confirm suitability by Valcan partners

10. COORDINATION AND QUALIFICATIONS

- All interfaces with windows, doors, movement joints and adjacent systems to be fully coordinated at detailed design stage.

- Final fixing layouts, bracket spacing and component selection subject to manufacturer’s calculations and approval.

- Installation to be carried out in accordance with manufacturer’s current installation guidance.

TECHNICAL DOWNLOADS

VitraDual Product Brochure April 2026

VitraDual (3mm) BBA Certification

VitraDual (3mm) Efectis Fire Classification Report

VitraDual (2-3mm) Warrington Fire Classification Report

VitraDual Rivet & Cassette CWCT Certification

VitraDual 5754 Grade CCPI Verification Details

VitraDual EPD Certificate

VitraDual Typical Drawings

VitraFix Product Brochure – July 2025

VitraFix Typical Drawings

ADDITIONAL RESOURCES & SUPPORT

| REGISTER FOR OUR FREE ACCREDITED CPD SEMINARS Valcan offer free CPD sessions throughout the year which are dedicated to sharing our experiences and helping you to make the correct choices for your building projects. The CPD program explains the safe cladding solutions available and how to comply with current regulations and illustrates the correct build ups to achieve the government standards. |  |
| --- | --- |
| ON-SITE TOOLBOX TALKS Valcan Toolbox Talks are delivered on site to support safe, compliant installation and reinforce best practice throughout the construction process. These sessions provide clear, practical guidance on system installation, handling, and building safety considerations, helping ensure consistent standards, informed operatives and a strong on‑site safety culture. |  |
| ORDER FREE PRODUCT SAMPLES Order up to three free samples from our website. See them up close, compare finishes, and find the perfect match for your next project with confidence. We offer colour-matching services for your specific needs. Provide us with the code or a sample and we'll match it while meeting your design requirements. |  |
| TECHNICAL SERVICES Valcan offer a wide range of technical services to support project design and system installation. In depth NBS Specifications, project specific CAD drawings, photographic overlays, U-values and condensation / humidity risk analysis documents are all readily available via our website. |  |
| QUALITY, ENVIRONMENTAL AND INFORMATION STANDARDS Valcan is certified to ISO 9001 and ISO 14001, confirming independently audited quality and environmental management systems that support consistent performance and responsible operations. The organisation is also CCPI Verified, demonstrating a commitment to clear, accurate and transparent construction product information in line with current building safety expectations. |  |

For more information visit www.valcan.co.uk/support/ or contact the team on:

01278 428245 or email enquiries@valcan.co.uk

FAIRVIEW EUROPE T/A VALCAN

Dunball House, Woodlands Court Business Park

Bristol Road, Bridgwater, Somerset TA6 4FJ

Telephone: 01278 428245

Email: enquiries@valcan.co.uk

Web: www.valcan.co.uk