Specifications for NBS Chorus

VitraDual & VitraFix (VF1/VF2) Rivet Fix System

SUMMARY

System name: VitraDual Rivet Fix Rainscreen System

System basis of design: VitraDual A1 aluminium rivet fix panels installed on the VitraFix carrier system.

Manufacturer: Valcan with approved partner system components

System type: A1 aluminium rivet fix rainscreen façade

Fixing method: Rivet Fix

Fire rating: A1 throughout (panels, carrier, insulation, membrane)

Warranty: Up to 30 years

Typical applications: Residential, commercial, high-rise

Standards: BS 8414 tested, CWCT, BBA certified*

* Vitradual 3mm is manufactured by Valcan and is available with BBA certification when produced at BBA-certified production facilities. Certification requirements must be specified at enquiry and order stage. Contact us for more details

CONTENTS

| VITRADUAL A1 ALUMINIUM RIVET FIX RAINSCREEN SYSTEM | P 4-5 |
| --- | --- |
| System description |  |
| Panel description |  |
| Performance |  |
| Finish and colour |  |
| Application |  |
| Specifiers notes – VitraDual | P 5 |
| VITRAFIX ALUMINIUM CARRIER SYSTEM OVERVIEW | P 6-7 |
| System description |  |
| Performance and compliance |  |
| System options |  |
| Application |  |
| TYPICAL DRAWINGS | P 8-09 |
| Rivet fix typical joint |  |
| Rivet fix horizontal joint |  |
| Rivet fix external corner |  |
| Rivet fix internal corner |  |
| Specifiers notes - VitraFix | P 7 |
| TECHNICAL SCHEDULE | P 10 – 14 |
| System summary |  |
| Primary cladding details |  |
| Panel Fixings and stiffening |  |
| Carrier System – Primary Support |  |
| Horizontal rails (if required) |  |
| Joints and tolerances |  |
| Cavity and ventilation |  |
| Ancillary components |  |
| Masonry fixings (if required) |  |
| TECHNICAL DOWNLOADS | P 15 |
| ADDITIONAL RESOURCES | P 16 |
| Contact details | P 17 |

VITRADUAL A1 ALUMINIUM RIVET FIX RAINSCREEN SYSTEM

SYSTEM DESCRIPTON

VitraDual is a non-combustible aluminium rivet fix rainscreen façade system, based on flat solid aluminium panels mechanically fixed using colour-matched rivets, installed onto an aluminium carrier system. The system is designed to provide a flat, durable and architecturally refined façade solution suitable for use on high-rise and low-rise buildings.

The system is eligible for a single-point, system-based warranty when supplied and installed in accordance with manufacturer’s recommendations.

PANEL DESCRIPTION

VitraDual panels are manufactured from solid aluminium and manufactured from solid aluminium flat panels mechanically fixed using colour-matched rivets.

- Material: Aluminium alloy (5754 marine-grade)

- Thickness:

  - 2.0 mm (rivet fixing only)

  - 3.0 mm

- Fire classification: Classified A1 in accordance with BS EN 13501-1

- Fixing method: Secret-fixed rivet system

- Surface flatness: Extremely flat finish with low surface stress and no oil-canning effect when designed and installed in accordance with manufacturer guidance.

Panels may be manufactured to bespoke sizes and formats, including curved or perforated panels, subject to project requirements and structural assessment.

PERFORMANCE

Fire performance: Non-combustible system suitable for use in buildings subject to ADB v2 requirements.

Wind loading: CWCT tested and accredited to a safety loading of 3600 Pa (subject to project-specific calculations).

Thermal Conductivity: 138 W/mK

Durability: High impact resistance and suitable for use in high-traffic locations.

Certification: 3mm BBA certified* (Certificate No. 20/5824).

Final system performance is subject to project-specific structural calculations and façade design.

* Vitradual 3mm is manufactured by Valcan and is available with BBA certification when produced at BBA-certified production facilities. Certification requirements must be specified at enquiry and order stage. Contact us for more details

FINISH AND COLOUR

Panels are factory finished with high-performance PVDF Kynar 500 or FEVE paint systems, providing excellent resistance to weathering, UV exposure and atmospheric pollutants.

Colour range: Standard and bespoke colours available.

Maintenance: Low maintenance compared to polyester-coated alternatives.

APPLICATION

The VitraDual system is suitable for use on both internal and external façades of commercial, residential and industrial buildings where a clean, modern architectural appearance is required.

Typical applications include:

- High-rise residential buildings

- Commercial offices

- Mixed-use developments

- Areas requiring robust, non-combustible façade materials

Specifiers notes:

Panel Layout: Panel sizes, joint widths and fixing centres to be confirmed in accordance with wind loading and CWCT guidance.

Fixing Strategy: Combination of fixed and sliding rivet points to allow for thermal movement. Sliding points to be installed using centralising tools in accordance with manufacturer’s guidance.

Wind Loading: Reference CWCT testing is to 3600 Pa safety load. Project-specific wind loads may require revised fixing centres or subframe configuration.

Cavity Depth: Minimum cavity depths to be maintained in accordance with CWCT guidance and fire strategy.

Finish Selection: Colour, gloss level and coating system to be specified at enquiry stage to determine warranty duration.

Warranty: Up to 30 years, subject to specification compliance and installation in accordance with Valcan guidance.

VITRAFIX ALUMINIUM CARRIER SYSTEM OVERVIEW

The following carrier system details describe a typical aluminium support system suitable for cassette-fixed A1 rainscreen façades, illustrated using the VitraFix carrier system.

SYSTEM DESCRIPTION

VitraFix is an aluminium carrier system comprising brackets, rails and fixings designed to support non-combustible rainscreen cladding systems. The system is tested to BS 8414-1 and BS 8414-2 and assessed by the Centre for Window and Cladding Technology (CWCT).

The system is designed to provide a cost-effective, durable and adjustable support solution suitable for use across a wide range of building types and substrates.

PERFORMANCE AND COMPLIANCE

Fire performance: Tested to BS 8414-1 and BS 8414-2 as part of an external wall system.

Structural assessment: CWCT tested (subject to project-specific calculations).

Durability: Aluminium components designed for long-term external use.

Warranty: A system warranty is available when specified and installed in accordance with manufacturer’s requirements.

SYSTEM OPTIONS

VitraFix VF1 – Primary Bracket System

VitraFix VF1 is an adjustable aluminium bracket system designed to accommodate variations in substrate tolerance and cavity depth.

- Adjustability: 102–235 mm

- Application: Direct bracket fixing to structural substrate

- Extensions: Additional depth may be achieved by incorporating the VitraFix VF2 system components.

- Options: Custom bracket depths are available subject to project requirements.

VF1 is typically used as the primary carrier system for cassette-fixed rainscreen façades.

VitraFix VF2 – Rail and Secondary Support System

VitraFix VF2 comprises aluminium or steel top-hats and C-channel rails designed to provide additional system depth or allow brackets to be installed independently of stud locations.

- Enables alignment of the carrier system where stud positions are restrictive

- Can be used to increase overall system build-up depth

- Top-hats may be used independently as an alternative to timber batten sub-grids

VF2 components may be used in conjunction with VF1 brackets or as part of a secondary support arrangement, subject to design requirements.

APPLICATION

The VitraFix carrier system is suitable for use with non-combustible rainscreen façade systems on residential, commercial and mixed-use buildings. Final system selection, bracket spacing and fixing strategy shall be determined by project-specific structural and wind load calculations.

Specifier’s Notes:

Bracket type, spacing and material selection to be confirmed based on substrate, cavity depth and wind load calculations.

Movement joints and tolerance allowances to be incorporated in façade design to accommodate thermal expansion, substrate movement and building deflection.

TYPICAL DETAILS

Details shown are indicative only and shall be developed to suit project‑specific geometry, substrate and fire strategy.

RIVET FIX DETAILS SHOWING TYPICAL JOINT

RIVET FIX DETAILS SHOWING HORIZONTAL JOINT

RIVET FIX DETAILS SHOWING EXTERNAL CORNER

RIVET FIX DETAILS SHOWING INTERNAL CORNER

All Technical Details can be found at https://valcan.co.uk/wp-content/uploads/2026/02/VitraDual-Typical-Details-2026.zip or on request.

TECHNICAL SCHEDULE

Purpose of this schedule

This Technical Schedule provides detailed system components and reference information to support detailed design, coordination and installation.

Final configuration, sizing and fixing arrangements shall be confirmed by project-specific calculations and manufacturer approval.

1. SYSTEM SUMMARY

- Façade system name: VitraDual Rivet Fix System

- System type: Aluminium rivet fix rainscreen façade system

- Primary carrier system: VitraFix VF1 / VF2

- Fire classification (overall system intent): A1

- Applicable standards: CWCT, BBA

(Informational summary only – see sections below for component detail)

2. PRIMARY CLADDING DETAILS

Product reference: VitraDual Rivet Fix

Manufacturer: Fairview Europe t/a Valcan

Product name: VitraDual

Material: 5754 grade aluminium

Thickness and Weight:

- Thickness: 3mm

- Weight (kg/m²): 8.13 kg/m².

Panel Dimensions:

- Indicative maximum panel sizes up to 5000 mm x 1500 mm, subject to wind loading and handling.

- Panels supplied as flat sheets.

Fire Performance:

- Classification to BS EN 13501-1: Classified A1

3. PANEL FIXING

3.1 PRIMARY PANEL FIXINGS

|  | Rail/fixing type | Reference | Sizes |
| --- | --- | --- | --- |
|  | Vitrafix A4 stainless steel rivet, colour matched to panel | VFR4.820C | 4.8 × 20 mm rivet |

- Edge distance: 16–50 mm

- Fixing centres: Max. 600 mm (wind load dependant)

3.2 ACCESSORIES

|  | Type | Reference |
| --- | --- | --- |
|  | Centralising Tool | VFCT74.9 |
|  | Rivet Nosepiece Adapter (to prevent over tightening) | VFNPR/16 |

4. CARRIER SYSTEM

Carrier system reference: VitraFix VF1

- Manufacturer: Fairview Europe t/a Valcan

Material: Aluminium

System Description: Brief description of role within façade build-up

|  | Bracket Reference | Size | Adjustment Range |
| --- | --- | --- | --- |
|  | VF140DB | 65 mm-260 mm double bracket | 142–172 mm |
|  | VF140SB | 65mm-260 mm single bracket | 142–172mm |

4.1 BRACKETS

Bracket selection and spacing subject to structural and wind load calculations.

4.2 THERMAL/ISOLATION PADS

|  | Bracket Reference | Size | Adjustment Range |
| --- | --- | --- | --- |
|  | VFIPDB | Double bracket thermal pad | 3 mm |
|  | VFIPSB | Single bracket thermal pad | 3mm |

5. HORIZONTAL RAILS (IF APPLICABLE)

System reference: VitraFix VF2

Purpose: VF2/U attached to SFS locations allowing brackets to be installed in isolation to stud location

Typical Applications: Not required for masonry

5.1 RAILS AND FIXINGS

|  | Rail/fixing type | Reference | Sizes |
| --- | --- | --- | --- |
|  | U-Rail | VF2/U | 90x15x2mm / 174x15x2mm |
|  | VitraFix Self Drill Hex head with washer | VFSD.538/A4 | 5.5x38mm |
|  | VitraFix Self Drill Hex head | VFSD4.825 (See manufacturers details for fixing centres) | 4.8x25mm, 8mm |

6. JOINTS AND TOLERANCES

Panel Joint Type:

- Typical joint width 8-10mm

- Open joints

7. CAVITY AND VENTILATION

Cavity Depth:

- Minimum 50 mm open‑joint installation, measured from face of insulation to rear of panel. May be reduced by up to 50% at fire barrier locations

Ventilation Strategy:

- Open-joint rainscreen in accordance with CWCT guidance

8. ANCILLARY COMPONENTS

8.1 SHEATHING BOARDS

- Product reference: ProcellaPro

- Panel size: 2500 x 1250mm

- Thickness/weight: 12 mm / 21.6 kg/m²

- Fire Rating to BS EN  13501-1:	A1

- Manufacturer: Fairview Europe t/a Valcan

8.2 FIXINGS

|  | Rail/fixing type | Reference | Sizes |
| --- | --- | --- | --- |
|  | VitraFix Self Drill ProcellaPro WT fixing | VFSDPP4.838/WT | 4.8x38mm |

8.3 FIRE BARRIERS

Product reference:

- Open state horizontal barrier

- Non-ventilated vertical barrier

Fire performance:

- Insulation:

- Integrity:

Manufacturer: Valcan partner manufacturer of fire barriers

Fixings: Fire tested fixings to be advised by Valcan to equivalent performance of fire barrier, fixing reference dictated by substrate.

8.4 BREATHER MEMBRANE

- Product reference: Valcan partner manufacturer Breather Membrane

- Fire performance (BS EN 13501-1): A1 or A2

8.5 INSULATION

- Product reference: Valcan partner manufacturer of insulation

- Thickness: TBC

- Thermal Performance: 0.034 w/mK

- Fire performance (BS EN 13501-1): A1

9. MASONRY FIXINGS (IF REQUIRED)

Masonry / Concrete Fixings:

- ETA-approved, fire-rated fixings

- Pull-out testing to confirm suitability by Valcan partners

10. COORDINATION AND QUALIFICATIONS

- All interfaces with windows, doors, movement joints and adjacent systems to be fully coordinated at detailed design stage.

- Final fixing layouts, bracket spacing and component selection subject to manufacturer’s calculations and approval.

- Installation to be carried out in accordance with manufacturer’s current installation guidance.

TECHNICAL DOWNLOADS

VitraDual Product Brochure April 2026

VitraDual (3mm) BBA Certification

VitraDual (3mm) Efectis Fire Classification Report

VitraDual (2-3mm) Warrington Fire Classification Report

VitraDual Rivet & Rivet CWCT Certification

VitraDual 5754 Grade CCPI Verification Details

VitraDual EPD Certificate

VitraDual Typical Drawings

VitraFix Product Brochure – July 2025

VitraFix Typical Drawings

ADDITIONAL RESOURCES & SUPPORT

| REGISTER FOR OUR FREE ACCREDITED CPD SEMINARS Valcan offer free CPD sessions throughout the year which are dedicated to sharing our experiences and helping you to make the correct choices for your building projects. The CPD program explains the safe cladding solutions available and how to comply with current regulations and illustrates the correct build ups to achieve the government standards. |  |
| --- | --- |
| ON-SITE TOOLBOX TALKS Valcan Toolbox Talks are delivered on site to support safe, compliant installation and reinforce best practice throughout the construction process. These sessions provide clear, practical guidance on system installation, handling, and building safety considerations, helping ensure consistent standards, informed operatives and a strong on‑site safety culture. |  |
| ORDER FREE PRODUCT SAMPLES Order up to three free samples from our website. See them up close, compare finishes, and find the perfect match for your next project with confidence. We offer colour-matching services for your specific needs. Provide us with the code or a sample and we'll match it while meeting your design requirements. |  |
| TECHNICAL SERVICES Valcan offer a wide range of technical services to support project design and system installation. In depth NBS Specifications, project specific CAD drawings, photographic overlays, U-values and condensation / humidity risk analysis documents are all readily available via our website. |  |
| QUALITY, ENVIRONMENTAL AND INFORMATION STANDARDS Valcan is certified to ISO 9001 and ISO 14001, confirming independently audited quality and environmental management systems that support consistent performance and responsible operations. The organisation is also CCPI Verified, demonstrating a commitment to clear, accurate and transparent construction product information in line with current building safety expectations. |  |

For more information visit www.valcan.co.uk/support/ or contact the team on:

01278 428245 or email enquiries@valcan.co.uk

FAIRVIEW EUROPE T/A VALCAN

Dunball House, Woodlands Court Business Park

Bristol Road, Bridgwater, Somerset TA6 4FJ

Telephone: 01278 428245

Email: enquiries@valcan.co.uk

Web: www.valcan.co.uk