Specifications for NBS Chorus

VitraDual Hook-on Cassette & VitraFix (VF1/VF2) System

SUMMARY

System name: VitraDual Cassette Hook-on Rainscreen System

System basis of design: VitraDual A1 aluminium cassette panels fabricated for hook-on installation and supported on the VitraFix VF1 aluminium carrier subframe system.

Manufacturer: Valcan with approved partner system components

System type: A1 aluminium cassette hook-on rainscreen façade

Fixing method: Hook-on cassette fixing using proprietary cassette hook brackets engaged onto vertical aluminium rails.

Fire rating: A1 throughout (panels, carrier, insulation, membrane)

Warranty: Up to 30 years

Typical applications: Residential, commercial, high-rise

Standards: BS 8414 tested, CWCT, BBA certified*

* Vitradual 3mm is manufactured by Valcan and is available with BBA certification when produced at BBA-certified production facilities. Certification requirements must be specified at enquiry and order stage. Contact us for more details

CONTENTS

| VITRADUAL A1 ALUMINIUM HOOK-ON CASSETTE RAINSCREEN SYSTEM | P 4-5 |
| --- | --- |
| System description |  |
| Panel description |  |
| Performance |  |
| Finish and colour |  |
| Application |  |
| Specifiers notes – VitraDual | P 6 |
| VITRAFIX ALUMINIUM CARRIER SYSTEM OVERVIEW | P 7-8 |
| System description |  |
| Performance and compliance |  |
| System options |  |
| Application |  |
| Specifiers notes - VitraFix | P 8 |
| TYPICAL DRAWINGS | P 9 |
| Hook-on Cassette Detail |  |
| TECHNICAL SCHEDULE | P 10 – 14 |
| System summary |  |
| Primary cladding details |  |
| Panel Fixings and stiffening |  |
| Carrier System – Primary Support |  |
| Horizontal rails (if required) |  |
| Joints and tolerances |  |
| Cavity and ventilation |  |
| Ancillary components |  |
| Masonry fixings (if required) |  |
| Typical complete system build up (illustration) | P15 |
| TECHNICAL DOWNLOADS | P 16 |
| ADDITIONAL RESOURCES | P 17 |
| Contact details | P 18 |

VITRADUAL A1 ALUMINIUM HOOK-ON CASSETTE RAINSCREEN SYSTEM

SYSTEM DESCRIPTON

VitraDual is a non-combustible aluminium rainscreen façade system, based on folded aluminium cassette panels with concealed fixings, installed onto an aluminium carrier system. The system is designed to provide a flat, durable and architecturally refined façade solution suitable for use on high-rise and low-rise buildings.

The system is eligible for a single-point, system-based warranty when supplied and installed in accordance with manufacturer’s recommendations.

PANEL DESCRIPTION

VitraDual panels are manufactured from solid aluminium and formed as folded cassette units. Panels are fabricated with rear returns and fixing geometry to allow engagement with proprietary hook-on cassette brackets fixed to the rear of the panel.

- Material: Aluminium alloy 5754 (marine grade)

- Thickness: 3.0 mm

- Fire classification: A1 to BS EN 13501-1

- Fixing method: Hook-on cassette system (secret-fixed)

Panels may be manufactured to bespoke sizes and formats, including curved or perforated panels, subject to project requirements and structural assessment.

PERFORMANCE

Fire performance: Non-combustible system suitable for use in buildings subject to ADB v2 requirements.

Wind loading: CWCT tested and accredited to a safety loading of 3600 Pa (subject to project-specific calculations).

Thermal Conductivity: 138 W/mK

Durability: High impact resistance and suitable for use in high-traffic locations.

Certification: 3mm BBA certified* (Certificate No. 20/5824).

Final system performance is subject to project-specific structural calculations and façade design.

* Vitradual 3mm is manufactured by Valcan and is available with BBA certification when produced at BBA-certified production facilities. Certification requirements must be specified at enquiry and order stage. Contact us for more details

FINISH AND COLOUR

Panels are factory finished with high-performance PVDF Kynar 500 or FEVE paint systems, providing excellent resistance to weathering, UV exposure and atmospheric pollutants.

Colour range: Standard and bespoke colours available.

Maintenance: Low maintenance compared to polyester-coated alternatives.

APPLICATION

The VitraDual system is suitable for use on both internal and external façades of commercial, residential and industrial buildings where a clean, modern architectural appearance is required.

Typical applications include:

- High-rise residential buildings

- Commercial offices

- Mixed-use developments

- Areas requiring robust, non-combustible façade materials

PANEL FIXING METHOD – HOOK-ON SYSTEM

VitraDual cassette panels are fixed using a proprietary mechanical hook-on system. Hook-on brackets are mechanically fixed to the rear of each cassette during fabrication and engage onto vertical aluminium carrier rails fixed to the VitraFix VF1 bracket system.

- Panels are installed by lowering into position from above and mechanically restrained as required

- Fixings are concealed within the cassette geometry

- No face fixings are visible

- System allows for thermal movement and substrate tolerances

Specifiers notes:

General: Specifier to confirm system configuration, panel dimensions and fixing strategy in conjunction with the manufacturer to suit project-specific structural, fire and environmental requirements.

Fire Performance: The VitraDual system is non-combustible and achieves a fire classification of A1 to BS EN 13501‑1. System build-up, including insulation, cavity barriers and breather membrane, shall be coordinated to ensure compliance with Approved Document B (ADB) and project fire strategy.

Panel Size and Layout: Maximum panel dimensions, joint spacing and cassette profiles are subject to wind loading, substrate condition and fixing centres. Final panel sizes shall be confirmed through project-specific calculations and manufacturer approval.

Wind Loading and Structural Design: Bracket type, spacing and panel support shall be determined based on site wind load calculations in accordance with CWCT guidance. Reference wind load testing is to 3600 Pa safety loading; higher or project-specific loadings may require design adjustment.

Finish and Colour Selection: Final colour, gloss level and coating system (PVDF Kynar 500 or FEVE) to be selected from the manufacturer’s standard or bespoke ranges. Colour selection may influence lead times and warranty duration.

Cavity Depth and Fire Barriers: Minimum cavity depths to be maintained in accordance with CWCT guidance, fire barrier requirements and airflow performance. Cavity depth reductions may be required at fire barrier locations and other pinch points and shall be coordinated at detailed design stage.

Thermal Bridging: Thermal isolators or pads may be required at bracket locations to limit cold bridging in accordance with project thermal performance requirements.

Warranty: Manufacturer warranty of up to 30 years is available, subject to system configuration, coating specification and compliance with Valcan installation guidance.

Coordination: Window interfaces, movement joints, flashings and terminations shall be detailed in coordination with adjacent façade elements to ensure continuity of fire performance, weather resistance and visual alignment.

VITRAFIX ALUMINIUM CARRIER SYSTEM OVERVIEW

The following carrier system details describe a typical aluminium support system suitable for hook-on A1 aluminium cassette rainscreen façades, illustrated using the VitraFix carrier system.

SYSTEM DESCRIPTION

VitraFix is an aluminium carrier system comprising adjustable brackets, vertical rails and fixings designed to support non-combustible rainscreen cladding systems, including hook-on cassette applications. The system is tested to BS 8414-1 and BS 8414-2 and assessed by the Centre for Window and Cladding Technology (CWCT).

When used in a hook-on cassette configuration, the carrier system provides a stable and adjustable support framework onto which vertical aluminium rails are fixed to allow engagement of proprietary hook-on cassette brackets.

The system is designed to offer a cost-effective, durable and highly adjustable support solution suitable for a wide range of building types, substrates and façade geometries.

PERFORMANCE AND COMPLIANCE

Fire performance: Tested to BS 8414-1 and BS 8414-2 as part of an external wall system.

Structural assessment: CWCT tested (subject to project-specific calculations).

Durability: Aluminium components designed for long-term external use.

Warranty: A system warranty is available when specified and installed in accordance with manufacturer’s requirements.

SYSTEM OPTIONS

VitraFix VF1 – Primary Bracket System

VitraFix VF1 is an adjustable aluminium bracket system designed to accommodate variations in substrate tolerance and cavity depth.

- Adjustability: 102–235 mm

- Application: Direct bracket fixing to structural substrate

- Extensions: Additional depth may be achieved by incorporating the VitraFix VF2 system components.

- Options: Custom bracket depths are available subject to project requirements.

VF1 is typically used as the primary carrier system for hook-on aluminium cassette rainscreen façades.

APPLICATION

The VitraFix carrier system is suitable for use with non-combustible rainscreen façade systems incorporating hook-on aluminium cassette panels on residential, commercial and mixed-use buildings.

Final system selection, bracket type, spacing, rail configuration and fixing strategy shall be determined by project-specific structural design and wind load calculations.

Specifier’s Notes:

Bracket type, spacing and material selection to be confirmed based on substrate condition, cavity depth and wind load requirements.

Vertical rail selection and fixing strategy shall be coordinated with the selected hook-on cassette fixing arrangement.

Movement joints and tolerance allowances shall be incorporated into the façade design to accommodate thermal expansion, substrate movement and building deflection.

Installation Notes:

VitraDual hook-on cassette panels shall be installed onto the VitraFix VF1 carrier system in accordance with the manufacturer’s current installation guidance.

Installation shall typically proceed as follows:

- VitraFix VF1 brackets are fixed directly to the structural substrate and aligned to achieve the required cavity depth and façade tolerances.

- Vertical aluminium carrier rails are fixed to the VF1 brackets in the required layout to receive hook-on cassette brackets.

- Proprietary hook-on brackets fixed to the rear of each cassette panel are engaged onto the vertical rails by lowering panels into position from above.

- Panels shall be located to achieve consistent joint alignment and restrained as required in accordance with the system design.

The hook-on system allows for concealed fixings, thermal movement of materials and accommodation of substrate tolerances. Provision shall be made for installation sequencing, tolerances and, where required, panel removal strategy at detailed design stage.

TYPICAL DETAILS

Details shown are indicative only and shall be developed to suit project‑specific geometry, substrate and fire strategy.

HOOK-ON CASSETTE DETAIL

All Technical Details can be found at https://valcan.co.uk/wp-content/uploads/2026/02/VitraDual-Typical-Details-2026.zip or on request.

TECHNICAL SCHEDULE

Purpose of this schedule

This Technical Schedule provides detailed system components and reference information to support detailed design, coordination and installation.

Final configuration, sizing and fixing arrangements shall be confirmed by project-specific structural calculations and manufacturer approval.

1. SYSTEM SUMMARY

- Façade system name: VitraDual Hook-On Cassette System

- System type: Aluminium hook-on cassette rainscreen façade system

- Primary carrier system: VitraFix VF1 (VF2 where required)

- Fire classification (overall system intent): A1

- Applicable standards: CWCT, BBA

(Informational summary only – see sections below for component detail)

2. PRIMARY CLADDING DETAILS

Product reference: VitraDual Hook-On Cassette

Manufacturer: Fairview Europe t/a Valcan

Product name: VitraDual

Material: 5754 grade aluminium

Thickness and Weight:

- Thickness: 3mm

- Weight (kg/m²): 8.13 kg/m².

Panel Dimensions:

- Indicative maximum panel sizes up to 5000 mm x 1500 mm, subject to wind loading, handling and cassette geometry.

- Depth – 20-40mm deep according to design

Fire Performance:

- Classification to BS EN 13501-1: Classified A1

3. PANEL FIXING AND STIFFENING

3.1 PRIMARY PANEL FIXINGS – HOOK-ON

|  | Rail/fixing type | Reference | Sizes |
| --- | --- | --- | --- |
|  | Hook-cassette bracket | VFHC/1 |  |
|  | VitraFix Self Drill Panhead A4, colour matched to panel | VFSDP5.535C | 5.5x35mm (200 box) |

Panel Stiffening:

- Stiffener type: 2.0 mm aluminium top‑hat

- Fixing method: Stud‑welded to reverse of cassette panel

- Final stiffener size and spacing subject to panel size and wind load calculations

4. CARRIER SYSTEM

Carrier system reference: VitraFix VF1

- Manufacturer: Fairview Europe t/a Valcan

Material: Aluminium

System Description: Primary adjustable aluminium carrier system supporting vertical rails for engagement of hook-on cassette panels.

|  | Bracket Reference | Size | Adjustment Range |
| --- | --- | --- | --- |
|  | VF140DB | 65 mm-260 mm double bracket | 142–172 mm |
|  | VF140SB | 65mm-260 mm single bracket | 142–172mm |

4.1 BRACKETS

Bracket selection and spacing subject to structural and wind load calculations.

4.2 THERMAL/ISOLATION PADS

|  | Bracket Reference | Size | Adjustment Range |
| --- | --- | --- | --- |
|  | VFIPDB | Double bracket thermal pad | 3 mm |
|  | VFIPSB | Single bracket thermal pad | 3mm |

4.3 RAILS

|  | Rail type | Reference | Sizes |
| --- | --- | --- | --- |
|  | T-Rail | VFTR503 / VFTR506 | 50 x 100 x 3000 mm / 50 x 100 x 6000 mm |
|  | L-Rail | VFLR503 / VFLR506 | 50 x 50 x 3000 mm / 50 x 50 x 6000 mm |

5. HORIZONTAL RAILS (IF APPLICABLE)

System reference: VitraFix VF2

Purpose: Enables bracket positioning independent of stud locations (typically SFS substrates)

Typical Applications: Not required for masonry substrates

5.1 RAILS AND FIXINGS

|  | Rail/fixing type | Reference | Sizes |
| --- | --- | --- | --- |
|  | U-Rail | VF2/U | 90x15x2mm / 174x15x2mm |
|  | VitraFix Self Drill Hex head with washer | VFSD.538/A4 | 5.5x38mm |
|  | VitraFix Self Drill Hex head | VFSD4.825 (See manufacturers details for fixing centres) | 4.8x25mm, 8mm |
|  | Masonry anchors - ETA approved fire-rated fixing | Valcan partner approved fixings |  |

6. JOINTS AND TOLERANCES

Panel Joint Type:

- Typically 20mm Shadow Gap

- Closed joints

7. CAVITY AND VENTILATION

Cavity Depth:

- Minimum of 25 mm closed joint installation - measured from face of insulation to rear of panel, can be reduced by 50% at fire barrier locations or other “pinch” points. (Check with insulation supplier on suitability of cavity depth for their product)

- [other] - consult CWCT section 3.4.4.4

Ventilation Strategy:

- Closed state

- Refer to CWCT guidance where applicable

8. ANCILLARY COMPONENTS

Ancillary panel fixings listed are for rear-of-panel fabrication only and do not represent face-fixed or rivet-fixed cassette installation.

8.1 SHEATHING BOARDS

- Product reference: ProcellaPro

- Panel size: 2500 x 1250mm

- Thickness/weight: 12 mm / 21.6 kg/m²

- Fire Rating to BS EN  13501-1:	A1

- Manufacturer: Fairview Europe t/a Valcan

8.2 FIXINGS

|  | Rail/fixing type | Reference | Sizes |
| --- | --- | --- | --- |
|  | VitraFix Self Drill ProcellaPro WT fixing | VFSDPP4.838/WT | 4.8x38mm |

8.3 FIRE BARRIERS

Product reference:

- Open state horizontal barrier

- Non-ventilated vertical barrier

Fire performance:

- Insulation:

- Integrity:

Manufacturer: Valcan partner manufacturer of fire barriers

Fixings: Fire tested fixings to be advised by Valcan to equivalent performance of fire barrier, fixing reference dictated by substrate.

8.4 BREATHER MEMBRANE

- Product reference: Valcan partner manufacturer Breather Membrane

- Fire performance (BS EN 13501-1): A1 or A2

8.5 INSULATION

- Product reference: Valcan partner manufacturer of insulation

- Thickness: TBC

- Thermal Performance: 0.034 w/mK

- Fire performance (BS EN 13501-1): A1

9. MASONRY FIXINGS (IF REQUIRED)

Masonry / Concrete Fixings:

- ETA-approved, fire-rated fixings

- Pull-out testing to confirm suitability by Valcan partners

10. COORDINATION AND QUALIFICATIONS

- All interfaces with windows, doors, movement joints and adjacent systems to be fully coordinated at detailed design stage.

- Final fixing layouts, bracket spacing and component selection subject to manufacturer’s calculations and approval.

- Installation to be carried out in accordance with manufacturer’s current installation guidance.

TYPICAL COMPLETE SYSTEM BUILD UP

Breather Membrane

ProcellaPro Sheathing Board

Vitradual Hook-on Cassette System

A1 Insulation

Vitrafix Subframe System Components

TECHNICAL DOWNLOADS

VitraDual Product Brochure April 2026

VitraDual (3mm) BBA Certification

VitraDual (3mm) Efectis Fire Classification Report

VitraDual (2-3mm) Warrington Fire Classification Report

VitraDual Rivet & Cassette CWCT Certification

VitraDual 5754 Grade CCPI Verification Details

VitraDual EPD Certificate

VitraDual Typical Drawings

VitraFix Product Brochure – July 2025

VitraFix Typical Drawings

ADDITIONAL RESOURCES & SUPPORT

| REGISTER FOR OUR FREE ACCREDITED CPD SEMINARS Valcan offer free CPD sessions throughout the year which are dedicated to sharing our experiences and helping you to make the correct choices for your building projects. The CPD program explains the safe cladding solutions available and how to comply with current regulations and illustrates the correct build ups to achieve the government standards. |  |
| --- | --- |
| ON-SITE TOOLBOX TALKS Valcan Toolbox Talks are delivered on site to support safe, compliant installation and reinforce best practice throughout the construction process. These sessions provide clear, practical guidance on system installation, handling, and building safety considerations, helping ensure consistent standards, informed operatives and a strong on‑site safety culture. |  |
| ORDER FREE PRODUCT SAMPLES Order up to three free samples from our website. See them up close, compare finishes, and find the perfect match for your next project with confidence. We offer colour-matching services for your specific needs. Provide us with the code or a sample and we'll match it while meeting your design requirements. |  |
| TECHNICAL SERVICES Valcan offer a wide range of technical services to support project design and system installation. In depth NBS Specifications, project specific CAD drawings, photographic overlays, U-values and condensation / humidity risk analysis documents are all readily available via our website. |  |
| QUALITY, ENVIRONMENTAL AND INFORMATION STANDARDS Valcan is certified to ISO 9001 and ISO 14001, confirming independently audited quality and environmental management systems that support consistent performance and responsible operations. The organisation is also CCPI Verified, demonstrating a commitment to clear, accurate and transparent construction product information in line with current building safety expectations. |  |

For more information visit www.valcan.co.uk/support/ or contact the team on:

01278 428245 or email enquiries@valcan.co.uk

CONTACTS

Façade Consultant: {Insert here}

Telephone: {Insert here}

Email:  {Insert here}

Internal Sales:  {Insert here}

Telephone:  {Insert here}

Email:  {Insert here}

FAIRVIEW EUROPE T/A VALCAN

Dunball House, Woodlands Court Business Park

Bristol Road, Bridgwater, Somerset TA6 4FJ

Telephone: 01278 428245

Email: enquiries@valcan.co.uk

Web: www.valcan.co.uk